# HKD Dual 43A H-Bridge Motor Driver (IBT-2) – Overview & Arduino Control

## Overview

The HKD Dual H-Bridge 43A motor driver (marked “V1326” / IBT_2) is a high-power DC motor driver module based on two Infineon BTS7960B half-bridge driver chips[1]. It’s designed to control a single brushed DC motor at 6–27 V up to about 43 A (peak)[2][3]. The module can handle high currents (43 A is a theoretical max; ~5–15 A continuous is more practical with proper cooling) and supports PWM frequencies up to 25 kHz for smooth speed control[4][5]. The BTS7960 drivers include built-in protection features – such as over-temperature, over/under-voltage shutdown, over-current limiting, short-circuit protection – and they provide a current-sense diagnostic output on the IS pins[6]. In short, the IBT-2 board is a robust solution for driving heavy DC motors forward and reverse. (Note: “IBT-2” is the module’s name, and BTS7960 refers to the H-bridge ICs it uses[7].)

The board has four large terminals for power and motor connections, typically labeled B+, B-, M+, M-. These are high-current screw terminals for the battery supply and the motor leads. It also has an 8-pin header for logic signals, labeled: RPWM, LPWM, R_EN, L_EN, R_IS, L_IS, VCC, and GND. Below we explain the purpose of each and how to connect them.

## Power Connections (Battery and Motor)

For the supply, connect B+ to the +12 V from your battery and B- to the battery’s ground/negative. (Ensure your 12 V battery’s ground is common to the Arduino ground as well.) The M+ and M- terminals go to the two leads of your DC motor[8]. If the motor runs opposite to your intended forward direction, you can swap M+ and M- (this just flips the motor polarity). The driver’s operating voltage range is 6–27 V, so a “12 V” lead-acid battery (≈13–14 V when fully charged) is well within spec[9].

Because this driver can source very high current from a large battery, be sure to use sufficiently thick wires for B+/B- and M+/M-, and consider adding an appropriate fuse or circuit breaker in line with the battery. A 100 Ah battery can deliver hundreds of amps into a short; although the BTS7960 will try to shut down on over-current, a fuse is important for safety[10]. The module has a large heat-sink to dissipate heat, but if you draw near the upper current limit, ensure good ventilation or additional cooling.

## Signal Pins (Control Interface)

The 8 small signal pins on the IBT-2 module are used to interface with your Arduino (or other microcontroller). Here’s a summary of each pin and how to use it:

- VCC – Logic supply input (5 V). This pin powers the driver’s internal logic (and its onboard 74HC244 buffer). Connect this to the 5 V output of your Arduino Nano[11]. (The driver accepts 5 V logic; 3.3 V is also recognized as HIGH per specs[12], but with an Arduino Nano you’ll use 5 V.) Note: Some IBT-2 boards can optionally use an onboard regulator, but typically you should supply 5 V directly from the Arduino[13].

- GND – Ground reference for logic. Connect this to the Arduino’s GND pin[14]. All grounds (Arduino, battery negative, and driver B-) must be common to ensure a proper reference for the control signals.

- RPWM – “Right PWM” input. This is one of the two PWM control pins for the H-bridge. Driving RPWM with a PWM signal (0–5 V) will command the motor speed/torque in one direction[15]. Connect RPWM to one of the Nano’s PWM-capable digital outputs (for example, D5 or D9)[11]. In use, you’ll apply a PWM signal here to run the motor in (say) the “forward” direction, while the other PWM pin (LPWM) is kept LOW (0% duty)[16]. A HIGH on RPWM corresponds to actively driving the motor in that direction.

- LPWM – “Left PWM” input. The second PWM control pin, for the opposite motor direction[15]. Connect LPWM to another PWM-capable output on the Nano (e.g. D6 or D10)[11]. To drive the motor in the opposite (“reverse”) direction, you apply PWM to LPWM while keeping RPWM LOW[16]. Essentially, RPWM and LPWM are complementary direction control signals – you activate one at a time depending on desired rotation direction, using PWM to control speed. Important: Never drive both PWM inputs high at the same time; only one should be active while the other is off (0) to avoid shorting the H-bridge[17].

- R_EN – “Right Enable” input. This is an enable pin for one half of the H-bridge (the half associated with the RPWM side). It’s an active-high inhibit/enable control: drive R_EN HIGH (5 V) to enable the output transistors for the “right” side, or LOW to disable them[15]. In normal operation you will keep R_EN enabled (HIGH). Many users simply tie R_EN to 5 V so that side is always on when the board is powered[18]. Alternatively, you can connect R_EN to an Arduino digital pin if you want to toggle it in code (e.g. for an emergency shutdown). If R_EN is pulled LOW, that half-bridge will turn off (no current to the motor from that side)[15].

- L_EN – “Left Enable” input for the other half of the H-bridge. It functions like R_EN, but for the LPWM/“left” side driver. L_EN must also be held HIGH to enable that side’s output[15]. You can tie L_EN to 5 V as well (common with R_EN) so that both halves are permanently enabled[19]. In practice, it’s typical to connect R_EN, L_EN, and VCC together to the Arduino’s 5 V supply so both sides are enabled continuously[11]. (This leaves the two PWM pins as the sole speed/direction control lines, simplifying control to “just 4 wires” from the MCU[20].)

- R_IS – “Right Current Sense” output. This is a diagnostic analog output from the BTS7960 chip on the R/PWM side. It outputs a voltage proportional to the load current, and can also flag fault conditions (like over-temperature or over-current shutdown)[21]. You do not need to connect R_IS for basic functionality; it can be left unconnected if you don’t require current feedback[14]. If you do want to use it, you could attach R_IS to an analog input on the Arduino to read the voltage. (Consult the BTS7960 datasheet for how to interpret the current-sense voltage; typically it outputs a small voltage proportional to the motor current when active.)

- L_IS – “Left Current Sense” output, analogous to R_IS but for the L/PWM half. This can also be left unconnected if not used[14], or wired to another analog input for current monitoring on that half. In normal operation, both R_IS and L_IS signals should stay low (near 0 V) until the motor is drawing significant current or a fault triggers a diagnostic signal[21].

Summary: For simple Arduino control, you will typically use 5 of the 8 pins: VCC, GND, RPWM, LPWM, and tie R_EN/L_EN to 5 V. The IS pins are optional. This configuration effectively needs only 4 Arduino connections (5 V, GND, and two PWM outputs) since R_EN and L_EN can be jumpered to VCC[20].

## Wiring to an Arduino Nano

Connecting the IBT-2 module to an Arduino Nano is straightforward. First, make sure the Arduino’s 5 V and GND are connected to the driver’s VCC and GND pins, respectively[11]. This powers the driver’s logic and establishes a common ground reference. Next, connect the two PWM control pins on the driver to two PWM-capable digital pins on the Nano. For example, you might use Arduino D5 for RPWM and D6 for LPWM (or any other PWM outputs you prefer)[11]. Then, either connect R_EN and L_EN to the 5 V rail (enabling both halves permanently) or wire them to two digital outputs on the Nano that you will set HIGH in setup()[18]. (Tying them directly to 5 V is simplest – you can even solder the R_EN and L_EN header pins to the VCC pin on the module, since all three need 5 V.) The current-sense pins (R_IS, L_IS) can remain unconnected unless you plan to use them.

Double-check that the battery’s B- (ground) is connected to the Arduino’s GND as well – this common ground is required for the driver to recognize the Arduino’s 5 V signals. At this point, your wiring should include: Battery to B+/B-, Motor to M+/M-, Nano 5V to VCC, Nano GND to GND, Nano D5→RPWM, Nano D6→LPWM, and R_EN + L_EN tied to 5 V. (If using different Arduino pins, adjust accordingly. Any PWM-capable pins will work for RPWM/LPWM, and any digital HIGH can serve for the EN pins if not tied directly.)

## Controlling the Motor (Arduino Code)

With the hardware connected, you can control the motor’s speed and direction using Arduino code by driving the RPWM/LPWM pins. The general strategy is: enable both halves (ensure R_EN and L_EN are HIGH or tied to 5 V), then use one of the PWM inputs at a time to drive the motor either forward or reverse. For example, to drive the motor forward, you might set the RPWM pin to a PWM value and ensure LPWM is set to 0 (LOW). To go reverse, do the opposite – set LPWM to a PWM value and RPWM to 0. In code, if you had RPWM_pin = 5 and LPWM_pin = 6 (per the earlier wiring), it would look like:

// assume RPWM_pin and LPWM_pin are set as outputs in setup()
analogWrite(RPWM_pin, speed);   // speed is 0-255 (0% to 100% duty cycle)
analogWrite(LPWM_pin, 0);       // forward rotation at "speed"

This drives the motor in one direction. For the opposite direction, you would do:

analogWrite(RPWM_pin, 0);
analogWrite(LPWM_pin, speed);   // reverse rotation at "speed"

When both PWM inputs are 0 (0% duty), the motor will coast to a stop (no driving current). For instance, setting analogWrite(RPWM_pin, 0); analogWrite(LPWM_pin, 0); effectively cuts power to the motor (freewheeling stop). If you instead drive both PWM pins HIGH at the same time, the driver would actively brake the motor by shorting its terminals – however, do not attempt this unless you understand the braking behavior, as the module’s internal logic might handle it differently. The safer method to stop is simply to set both PWM outputs to 0 (or disable the EN pins) to avoid any risk of conflicting signals.

Important: Only one of the two PWM inputs should be active at any given time (the other should be LOW/off). Never output a PWM signal to both RPWM and LPWM simultaneously, as that would command both halves of the H-bridge on and potentially short the supply[17]. Always ensure that if you want to drive forward, the reverse PWM line is set to 0 (and vice versa).

Because the Arduino Nano uses 8-bit PWM (analogWrite values 0–255), you can easily control speed. A value of 0 means stop (no drive), and 255 means full speed. Intermediate values provide proportional duty cycle drive. The BTS7960 drivers are efficient and have fast switching, so they can handle the Nano’s default PWM frequency (~490 Hz on most pins) easily, and you can increase the PWM frequency if needed (up to 25 kHz supported[4] to reduce any audible motor whine).

Example: If you wanted to spin the motor forward at ~50% speed, you could do analogWrite(RPWM_pin, 128) and analogWrite(LPWM_pin, 0). To reverse at 75% speed, do analogWrite(RPWM_pin, 0) and analogWrite(LPWM_pin, 192). Remember that R_EN and L_EN should be HIGH (enabled) during this operation – if you tied them to 5 V, they’re already taken care of. If you connected them to Arduino pins, set those pins HIGH in setup() before trying to drive the motor.

## Alternate Control Method (Optional)

It’s worth noting that the IBT-2 module can also be driven in an alternate way by swapping the roles of the PWM and EN pins, if you prefer a “direction + speed” control scheme. In this approach, you hold the RPWM/LPWM pins as static HIGH/LOW signals to choose direction, and instead apply a single PWM to the EN pins (both tied together) to control speed[22][23]. For example, to go forward you would set RPWM=HIGH, LPWM=LOW (selecting direction), then PWM the enable pins. To reverse, set RPWM=LOW, LPWM=HIGH, and use the same PWM signal. This method uses only one PWM output from the Arduino (for speed) plus two digital outputs (for the direction bits R/LPWM). It can work because the BTS7960’s R_EN/L_EN inputs can accept PWM as well[22]. However, the naming on the module is confusing – and the manufacturer’s typical usage is the one described earlier (PWM on RPWM/LPWM and EN tied high)[5]. If you’re just getting started, it’s recommended to use the standard method as described in the sections above. Both approaches ultimately achieve the same forward/reverse control – just be consistent and do not mix the two methods.

## Conclusion

To summarize, the HKD Dual H-Bridge 43A (IBT-2) driver is a powerful module for running a 12 V DC motor in both directions. You will wire B+ to your 12 V battery (which can be continuously charged, ~13.8 V) and B- to ground, attach the motor to M+/M-, and connect the 8-pin header to your Arduino Nano (5 V, GND, and the control pins). Tie R_EN and L_EN high (5 V) and use two Arduino PWM outputs for RPWM and LPWM to control speed and direction[11]. In code, drive one of these PWM pins at a time to make the motor go forward or reverse[16]. The driver’s built-in protections will help prevent damage from over-current or over-temp, but you should still fuse the battery and avoid overloading the motor continuously[10][6]. With this setup, you can reliably drive your 12 V, 5 A (continuous) motor forward and backward as needed. Enjoy your motor control project, and stay safe with those high currents!

Sources: Proper use and pin connections for the IBT-2 (BTS7960) driver[11][15]; module specifications and features[3][5]; example Arduino wiring and code for bidirectional motor control[16][17]; manufacturer’s notes on using 4 control lines[20].

[1] [7] [10] IBT_2 BTS7960 Motor Board — DCC-EX Model Railroading documentation

https://dcc-ex.com/reference/hardware/motorboards/IBT_2-motor-board-setup.html

[2] [5] [9] [13] [20] HKD DUAL H-BRG MOTOR DRIVER 43A - Communica [Part No: HKD DUAL H-BRG MOTOR DRIVER 43A]

https://www.communica.co.za/products/hkd-dual-h-brg-motor-driver-43a?srsltid=AfmBOoo21Ts6hSxPpOWrFVrWQDQbySmiDjfOkVELXjr25GHqDWJmZeOb

[3] [4] [6] [11] [12] [14] [16] [18] handsontec.com

https://www.handsontec.com/dataspecs/module/BTS7960%20Motor%20Driver.pdf

[8] [22] [23] Large Current Motor Driver BTS 7960 Module - alselectro

https://alselectro.wordpress.com/2021/11/08/6632/

[15] [21] BTS7960 BTN7960N IBT_2 motor drive module - Motors, Mechanics, Power and CNC - Arduino Forum

https://forum.arduino.cc/t/bts7960-btn7960n-ibt-2-motor-drive-module/1142926

[17] Overcurrent in motors driven by IBT2 - Motors, Mechanics, Power and CNC - Arduino Forum

https://forum.arduino.cc/t/overcurrent-in-motors-driven-by-ibt2/1141229

[19] DC motor with encoder to Arduino: restonweb — LiveJournal

https://restonweb.livejournal.com/476421.html